Exp No: 16

SERIAL INTERRUPT USING UART KEYBOARD INPUT IN KEIL

AIM:

To write and simulate an Embedded C program using serial interrupt in 8051 microcontroller such that while a message is continuously transmitted, any key pressed from the keyboard generates an interrupt and is displayed through UART.

APPARATUS / SOFTWARE REQUIRED:

- PC with Windows OS

- Keil μVision IDE

- 8051 Simulator (Keil built-in)

- Serial Window (UART #1)

THEORY:

The 8051 microcontroller provides a serial interrupt that occurs when:

- RI (Receive Interrupt flag) is set after receiving a character, or

- TI (Transmit Interrupt flag) is set after transmission.

In this experiment:

- UART is configured in Mode 1 (8-bit UART)

- Timer 1 is used to generate baud rate (9600)

- Serial interrupt is enabled using ES bit

- When a key is typed in the Keil Serial Window, the received character sets RI

- The Serial Interrupt Service Routine (ISR) is executed immediately

This demonstrates interrupt-driven serial communication, which is more efficient than polling.

PROGRAM:

#include <reg51.h>

unsigned char received_char;

bit receive_flag = 0;

/* Serial Interrupt Service Routine */

void serial_ISR(void) interrupt 4

{

if (RI)

{

RI = 0;

received_char = SBUF;

receive_flag = 1;

}

if (TI)

{

TI = 0;

}

}

/* Send one character through UART */

void send_char(unsigned char ch)

{

ES = 0;          /* Disable serial interrupt temporarily */

SBUF = ch;

while(TI == 0);

TI = 0;

ES = 1;          /* Enable serial interrupt */

}

/* Send a string through UART */

void send_string(char *str)

{

while(*str)

{

send_char(*str);

str++;

}

}

/* Delay */

void delay(void)

{

unsigned int i;

for(i = 0; i < 50000; i++);

}

void main(void)

{

/* UART configuration */

SCON = 0x50;       /* UART Mode 1, 8-bit, REN enabled */

TMOD = 0x20;       /* Timer 1, Mode 2 */

TH1 = 0xFD;        /* 9600 baud rate */

TR1 = 1;           /* Start Timer 1 *

/* Enable serial interrupt */

EA = 1;

ES = 1;

/* Initial value */

TI = 0;

while(1)

{

/* Continuously transmit message */

send_string("Hello World\r\n");

/* Check whether a key was received */

if(receive_flag)

{

receive_flag = 0;

send_string("\r\n>>> INTERRUPT RECEIVED: ");

send_char(received_char);

send_string("\r\n");

}

delay();

}

}

OUTPUT :

RESULT :

Thus the serial interrupt using UART was Successfully implemented and verified in keil simulator.