Exp No: 17

UART INTERFACING USING INTERRUPT IN KEIL

AIM:

To receive serial data using interrupt.

APPARATUS REQUIRED:

- PC with Windows OS

- Keil μVision IDE

- 8051 Simulator (Keil built-in)

- Serial Window (UART #1)

PROGRAM:

#include <reg51.h>

void serial_ISR(void) interrupt 4

{

if(RI)

{

P1 = SBUF;

RI = 0;

}

}

void main()

{

TMOD = 0x20;

TH1 = 0xFD;

SCON = 0x50;

TR1 = 1;

ES = 1;

EA = 1;

while(1);

}

OUTPUT:

a) Before Interruption

b) After interruption

RESULT :

Thus the serial data was received using interrupt and the received data was verified.